RAVAN-512: Multi-core Cryptographic Hardware Engine

Overview

RAVAN-512 is a high-performance 512-bit cryptographic engine designed using Verilog, targeting secure and scalable hardware implementations. The architecture is optimized for high throughput, efficient resource utilization, and robust multi-clock domain operation.

The design implements a 21-round encryption/decryption pipeline with dynamic key slicing and supports full RTL-to-GDSII flow validation.

Key Highlights

- 512-bit multi-core cryptographic architecture

- 21-round pipelined encryption/decryption engine

- Achieved timing closure at 500 MHz (2.0 ns) with +1.15 ns slack (45nm)

- Maximum theoretical frequency: ~1.26 GHz

- ~11.5K standard cells, ~29K µm² area

- Total power: 1.72 mW (Leakage: 7.59 µW)

- Multi-clock domain design with Gray-coded asynchronous FIFO

- Integrated AMBA AXI4/AHB interfaces

- End-to-end RTL-to-GDSII flow using OpenLane (Sky130)

Architecture

- Multi-core encryption datapath

- Dynamic key slicing unit (512-bit → 8 × 64-bit slices)

- 21-round transformation engine

- Control FSM for encryption/decryption flow

- Clock domain crossing (CDC) using asynchronous FIFO

Verification Strategy

RTL Verification:

- SystemVerilog/UVM-based constrained random verification

- Functional validation across multiple scenarios

Python Reference Model:

- High-level golden model for encryption/decryption

- Used to validate RTL outputs

- Faster functional validation before simulation

Physical Design & Implementation

- Synthesized using Cadence Genus (45nm)

- Achieved timing closure with positive slack

- RTL-to-GDSII flow using OpenLane (Sky130)

Results

Timing:

- Frequency: 500 MHz

- Slack: +1.15 ns

Area:

- ~11.5K standard cells

- ~29K µm²

Power:

- 1.72 mW total

- 7.59 µW leakage

Tools & Technologies

- Verilog, SystemVerilog, UVM

- Cadence Genus, Xcelium

- OpenLane, Yosys, Verilator

- Python (Reference Model)

Author

Devaprasath Thangavelu

RTL / ASIC Design Engineer